8-CHANNEL ZIGBEE

IRRIGATION CONTROLLER

8-zone irrigation controller

User Manual

ESP32-C6-Zero  •  Zigbee/Home Assistant  •  LCD 16×2

# 1. Overview

The 8-channel Zigbee irrigation controller is an open-source 8-zone irrigation controller built on the ESP32-C6-Zero. It allows solenoid valves (EV) to be activated manually, sets an independent timer for each zone, and runs an automatic sequence that activates them in order. The device integrates with Home Assistant via Zigbee.

## 1.1  Key features

- 8 independent relay outputs (EV1–EV8)

- Per-zone timer: 0 to 60 minutes

- Automatic sequence: activates zones in order, each for its configured time

- 16×2 LCD display with menu navigated via 3 buttons

- RGB status LED

- Zigbee connectivity with Home Assistant

- Factory reset via BOOT button

# 2. Hardware

## 2.1  Buttons

The device has three buttons for menu navigation:

| Button | Position | Function |
| --- | --- | --- |
| ◄ (Left) | Left | Go back / decrement value |
| OK | Centre | Confirm / enter submenu |
| ► (Right) | Right | Advance / increment value |

If the display is in sleep mode, the first press of any button only wakes the display without executing any action.

## 2.2  LCD display

The 16×2 LCD display shows device status and the configuration menu. It turns off automatically after 20 seconds of inactivity to reduce power consumption. Backlight and display re-activate on the first button press.

## 2.3  Status LED

The onboard RGB LED indicates the operational state of the device:

| Colour | Meaning |
| --- | --- |
| Blue | Searching — device is looking for a Zigbee network |
| Green | Connected — no active zone |
| Purple | Zone manually active |
| Yellow | Automatic sequence running |
| White | Factory reset in progress |

# 3. Home screen

The HOME screen is the main view, shown at startup and after inactivity. It changes appearance based on device state.

## 3.1  All off

No active zone. The second row shows the Zigbee connection state.

All off Online OK=menu

or, if not yet connected:

All off Search OK=menu

## 3.2  Manual zone active

When one or more zones are manually active, the first row shows the zone bitmask (1 = active, 0 = off). Zones are listed in order from EV1 to EV8, left to right.

EV: 10100000 OK=menu

In this example: EV1 and EV3 are active, all others off.

## 3.3  Sequence running

During the automatic sequence, the current zone and its configured time are shown.

SEQ: EV3 t=8min OK=menu

# 4. Main menu

Press OK from the HOME screen to open the main menu. Use ► to scroll items and OK to select. Press ◄ to return to HOME.

Start sequence < back OK>

Available menu items:

| Item | Description |
| --- | --- |
| Start sequence | Starts the automatic sequence. If the sequence is already running, the item becomes Stop sequence to stop it. |
| Timer settings | View and modify the timer of each zone. |
| Manual control | Activate or deactivate each zone individually. |

If a sequence is running, 'Timer settings' and 'Manual control' are disabled. Stop the sequence first to access them.

# 5. Timer settings

Each zone has an independent timer (0 to 60 minutes) that determines how long it stays active during the automatic sequence. A timer set to 0 excludes the zone from the sequence.

## 5.1  Navigation

From the main menu select Timer settings with OK. Use ◄ / ► to move between zones. Press OK to enter edit mode for the selected timer.

Timer EV1: 5min <back OK>

On the first zone (EV1), the ◄ button shows 'back' because it returns to the main menu.

## 5.2  Editing a timer

After pressing OK on the desired zone, edit mode is entered. The value in brackets is the one being changed.

EV1:[ 5] min <-min +min> OK

- ◄  decrements the value by 1 minute

- ►  increments the value by 1 minute

- OK  saves the value and returns to zone selection

The value is saved to non-volatile memory (NVS): it is retained even after power-off.

# 6. Manual control

Manual control allows activating or deactivating each zone independently of the automatic sequence, with no time limit.

## 6.1  Navigation

From the main menu select Manual control with OK. Use ◄ / ► to move between zones. Press OK to toggle the zone state (ON ↔ OFF).

EV2: ON < EV OK=tog

A zone stays active until manually turned off, until the automatic sequence is started (which deactivates all manual zones), or until the device is restarted.

# 7. Automatic sequence

The automatic sequence activates zones in order (EV1 → EV2 → … → EV8), each for the time set in its timer. Zones with timer = 0 are skipped.

## 7.1  Starting

- From the main menu select Start sequence with OK.

- The device turns off any manually active zones.

- The first zone with timer > 0 is activated.

- The LED turns yellow.

If all timers are zero, the sequence does not start and a warning is shown on the display.

## 7.2  Progression

When a zone's timer expires, the device automatically turns it off and activates the next one. The HOME screen shows the current zone and its time.

## 7.3  End and stop

- The sequence ends automatically after the last zone.

- To stop it manually: open the main menu → the item shows Stop sequence → press OK.

- At the end, the LED returns to green (if connected) or blue (if searching).

# 8. Zigbee / Home Assistant integration

## 8.1  First connection

At first power-on, or after a factory reset, the device automatically enters search mode (blue LED). In Home Assistant, open ZHA (Zigbee Home Automation) and start the add device procedure. The device is detected automatically.

## 8.2  Available entities

Once added, the device exposes the following entities in Home Assistant:

| Entity | Type | Description |
| --- | --- | --- |
| EV1 – EV8 (On/Off) | Switch | Activate/deactivate each zone individually |
| EV1 – EV8 (Timer) | Number (0–60) | Timer in minutes for the automatic sequence |
| Sequence | Switch | Start/stop the automatic sequence |

## 8.3  Automatic reconnection

If the Zigbee connection is lost, the device automatically retries every few seconds. No manual intervention is required. Zones remain in their current state during reconnection.

# 9. Zigbee factory reset

Factory reset clears the Zigbee network settings (channel, PAN ID, security keys) and returns the device to an 'unassociated' state. Zone timers are preserved.

## 9.1  Procedure

- Hold the BOOT button (on the ESP32-C6-Zero module) for at least 5 seconds.

- The LED turns white: reset is in progress.

- The device restarts automatically and returns to search mode (blue LED).

- In Home Assistant, remove the old device from ZHA and re-add the new one.

Warning: after factory reset the device is no longer recognised by Home Assistant. Remove the old device from the ZHA interface before re-adding the reset one.

# 10. Operating notes

## 10.1  Action priority

In case of simultaneous commands:

- A Zigbee command from Home Assistant has the same priority as a display command.

- Starting the sequence deactivates all manually active zones.

- A manual command on a zone during the sequence does not interrupt the sequence itself.

## 10.2  Timer memory

Timer values are saved to non-volatile memory (NVS) and survive power-off. They are cleared only by manually erasing the NVS partition (not by the Zigbee factory reset).

## 10.3  Display sleep

The display turns off automatically after 20 seconds of inactivity. To wake it, press any button: the first press only wakes the display without executing any action.

## 10.4  Power supply

The device accepts an external supply of 6 to 24 V for the relay coils and solenoid valves. The ESP32-C6-Zero module runs internally at 3.3 V via its onboard regulator.